- GENERAL

      - SUMMARY

        - This Section specifies administrative and procedural requirements for Alternates.

        - Definition:  An Alternate is an amount proposed by Bidders and stated on the Proposal Form for certain construction activities defined in the Bidding Requirements that may be added to or deducted from Base Bid amount if the Owner decides to accept a corresponding change in either the amount of construction to be completed, or in the products, materials, equipment, systems or installation methods described in Contract Documents.

        - Coordination:  Coordinate related Work and modify or adjust adjacent Work as necessary to ensure that Work affected by each accepted Alternate is complete and fully integrated into the project.  Costs for the coordination, modification, or adjustment necessary for each alternate are included in the costs for each Alternate.

      - SCHEDULE OF ALTERNATES

        - Add Alternates:	NONE

        - Deduct Alternates:

          - DEDUCT ALTERNATE NO. 1:  In lieu of complete subfloor removal, after removal all layers of flooring (VCT, VAT, linoleum, carpet, luan, ¼” plywood, ceramic tile, cement board, etc.) down to the original ¾” plywood subfloor, the Contractor shall Encapsulate exposed plywood subfloor surfaces with white tinted encapsulant, FiberlockAsbestos Building Compound, Aramsco Chemsafe 500W, or Foster 32-32 within a negative pressure enclosure and in accordance with N.J.A.C. 5:23-8.15.  After satisfactory final inspection per N.J.A.C. 5:23-8, the Contractor shall install and secure with screws ¼” plywood subfloor from the removed areas.

- END OF SECTION 01 23 00 -